Clay etiquette, processes and states

## Clay etiquette

- Keep all clay in the art classroom. - Only use clay for making artwork. - Keep clay in an airtight plastic bag. - Put your initials on your work. - Clean up clay with a damp cloth.

## Clay processes: making strong clay joins

- Score (roughen) the join surfaces with a tool (e.g. fork). - Apply a small amount of slip (clay and water mix) to scored surfaces with a tool (e.g. plastic knife). The slip works like glue. - Push and twist join surfaces together firmly. - If needed, add a coil of clay to smooth over the join with fingers. - Smooth join and coil neatly with fingers and a tool (e.g. plastic knife) until you cannot see the join.

## Clay states

| Raw clay | - clay in its natural state or clay that is not capable of being used without further preparation - may be wet or dry, clean or containing foreign matter |
| --- | --- |
| Slip | - the liquid form of clay - used to join clay surfaces and for decorative purposes, such as slip trailing and slip glazing - “engobe” is the term used to describe slips that have been coloured with oxides |
| Slurry | - a thicker form of slip, verging on the plastic state |
| Plastic | - the stage at which most clay is worked - sufficiently pliable and self-supporting to allow modelling, throwing and other forms of manipulation |
| Leatherhard | - plastic clay that has dried to a stage where it can be handled without deforming - retains original colour and is able to be bent to some extent without breaking - still soft enough to be carved, stamped, incised, and joined with slip |
| Greenware | - stage at which the clay has lost most of its plasticity and water through evaporation - does not feel cold to touch, has undergone a colour change, a weight loss and shrinkage through drying - cannot be effectively joined with slip, but can be scraped, sanded and incised - stage to which clay wares are dried before bisque firing |
| Bisque or biscuit | - clay that has been through a firing in 600ºC or higher — the higher the temperature, the more vitreous and the less porous the body - form is permanent and rock-like and cannot be converted back to plastic clay |
| Glaze or gloss | - clay body with a glaze or gloss melted over its surface - normally put on a bisque-fired body |